README.DOC - Lite Fax Software for OS/2

Copyright (c) 1992-1995 by Keller Group Inc.  All rights reserved.

Topics contained in this README.DOC file:

MODEM CONFIGURATION SETTINGS

CONDENSED USER'S GUIDE

MODEM CONFIGURATION SETTINGS

----------------------------

Tested modems are listed below.  Unless the manufacturer has changed

the product since the time of our testing, you should use the

following information to determine correct configure values for your

fax hardware.  Make these settings on the Modem Type page in the fax

program's Settings notebook.

(Many other modems will also work, but the vendors have not provided

us with evaluation units for testing.)

Note that manufacturers are constantly changing their modem hardware

and firmware (EPROMs).  If you suspect a problem, please contact your

modem's manufacturer for assistance in determining the exact version

of your hardware and EPROM.  The manufacturer may ask you to use a

data communications program and issue the ATI3 command to obtain

modem version information.

MANUFACTURER  MODEL                          CONFIGURATION

------------- ------------------------------ ----------------------------------

Adtech Micro  Externally Yours! (ROM v2.2)   Class-2, might need 'No DLE esc'

Angia         Safejack Terbo 19.2 Data/Fax   Class-1

(PCMCIA card) Flash ROM v 1.07

Apex          PCMCIA modem                   Class-1

ATI           14,400 Internal Fax/Modem      Class-1

AT&T          Keep In Touch PCMCIA           Class-1

AT&T Paradyne DataPort Fax Modem             Class-1

Austin Cmptrs Desktop fax modem              Class-2, Special command &Q6

Best Data     Smart One Fax 14,400 Modem     Class-1

Boca          Bocamodem 14.4Kbps  user reported: Class-1, Special command &C1

BIT Blitzer   XM124S/RFAX                    Class-2

Brooktrout    TR112, TR114, Gemini           Supported in full retail version

Calculus      EZ-FAX                         NOT COMPAT: Proprietary fax cmds

Complete PC   Complete Modem Plus            Class-1

Complete PC   certain models                 NOT COMPAT: Proprietary fax cmds

CompuCom      Speedmodem Combo (ROM v2.77)   Class-2

CPI           VIVA 14.4k internal            Class-1,Special command &C1

Dallas        Dallas Fax 14.4E Pro Plus      NOT COMPAT: Class 1 train failure

Try at 9600bps and special command &C1 or &C1\J0\Q3W0

Dataplex      DPX-223-55 2400/9600 rom5.42   Class-2

Dataplex      DPX-225-01 14,400 (ROM 1.00)   Class-2

DELL          Digitan (Dell desktop units)   Class-2,Xon recv start,no DLE esc

DELL          Datarace (Dell laptop units)   Class-1, Special command \J0\Q2

Digicom (DSI) Scout Plus, Spirit II          Class-1

Digicom (DSI) Digicom 9624LE                 NOT COMPAT: SendFax non-spec

Digicom (DSI) Digicom Connection 14.4+FAX    NOT COMPAT

Dove          DoveFax (ROM v4.2)             Class-1

Dr. Neuhaus   FURY 9600, 9600 TI, 14.4 TI    Class-2

Dr. Neuhaus   FURY 2400 POCKET/FAX, PC/FAX   Class-2

Dr. Neuhaus   FURY 2400 PC JUNIOR            SendFax

Dr. Neuhaus   FURY 2402 TI/FAX               SendFax

Elsa          2460, MicroLink 14.4T/TM/TL    Class-2, Special command \Q1

14.4T with ROM 2.01 also needs Special command &K1&R7

Everex        EverFax 24/96 (COM1/2 only)    Cls2,Alt bit order,Xon recv start

Formosa       MoFax MF-96P                   NOT COMPAT: SendFax train failure

Frecom        Fax96                          NOT COMPAT: Non-standard chip set

GammaLink     CPi, CP, CP MC, XPi            Supported in full retail version

GammaLink     XP, XP MC                      NOT COMPAT: Not externally prog.

Gateway       Telepath (with recent EPROM)   Class-2

Supported only with recent EPROMs.  Some older EPROM versions

(1992, early 1993) are incompatible.

Gateway       Telepath II                    Same as U S Robotics Sportster

GVC           FAXMODEM 9624Vbis              Class-1 or Class-2

Hayes         Accura 14.4                    Class-1

Hayes         Optima 24/96                   Class-1

Hayes         Optima 96/96                   Class-1

Hayes         Optima 14.4/14.4               Class-1

IBM           L40SX laptop modem             Class-1, No BPS s/d, Skip TCF read

Tested and shows limited compatability potential.  Even after

engineering changes made by IBM, reception and sending of faxes

have been reported as spotty and line-sensitive. For

alternatives, see Microsource Fax42.L40.

IBM           9600 PCMCIA Fax Modem          Class-1

IBM           14400 PCMCIA Fax Modem         Class-1 (EPROM v028 or higher)

IBM           Microelectronics 14.4/14.4     Class-2

IBM           Fax Concentrator               NOT COMPAT: Proprietary fax cmds

Image Comm    TWINCOM 24/96                  Class-2

Image Comm    TWINCOM Commuter 24/96         Class-1

Image Comm    TWINCOM 14,400 v1.13C          Class-2

Infotel       1414 VQE                       Class-2

Intel         144/144e or 144/144i V1.7-2.1  Class-2, some fail, try 9600 bps

Intel         144/144e or 144/144i V2.2      Class-1 (or 2), use Cls1, 9600bps

Intel         SatisFAXtion classic           Supported in full retail version

Intel         SatisFAXtion/100               Class-1

Intel         SatisFAXtion/200               Supported in full retail version

Intel         SatisFAXtion/300               Class-1

Intel         SatisFAXtion/400 (internal)    Supported in full retail version

Intel         SatisFAXtion/400e (external)   Class-1

Intel         Connection CoProcessor         NOT COMPAT: Proprietary fax cmds

Megahertz     96/96 FAX/Modem (P296FMV)      Class-1, Special command \J0\Q1

Megahertz     14400 Fax/Modem (XJ144)        Class-1

Megahertz     14400 Fax/Modem (XJ244)        Class-2,Xon recv start,No DLE esc

Megahertz     14400 Fax/Modem (XJ1144)       Class-1, Special command &K4

MICC          9642                           Class-2

MicroCom                      user reported: Class-1, Special command &C1

Microsource   Fax42.L40 (for IBM L40)        Class-2

Marketed as a replacement for the IBM L40 LAPTOP modem.

The modem has been tested and is compatible with the fax

software. (404 952-3336)

MultiTech     MultiFaxPC MT932               Class-2, Alt bit order

MultiTech     Multi Modem MT932EAF           Class-2, Alt bit order

MultiTech     Multi Modem II                 Class-2, Alt bit order

MultiTech     Multi Modem V.32 Fax           Class-2, Alt bit order

NetComm       M4F or M7F (ROM rev K)         Cls-2, Special cmd +FDCC=,3,0,2,0

Omnitel       9624+S/R                       NOT COMPAT: SendFax recv failures

Omnitel       Omnitel v1.10                  NOT COMPAT: Class-2 recv failures

P.C. Logic    Fax144EX                       Class-1 (recommended) or Class-2

P.C. Logic    Fax 24I                        Class-2, XON recv start

Practical     2400F MINI                     Class-1

Practical     2400FX96                       Class-2

Practical     2400 SendFAX                   SendFax

Practical     14400FXSA V.32bis              Class-1, Check TCF signal

Practical     14400FXMT V.32bis              Class-1 or Class-2

Prodigy       Eagle                          Class-1

PureData      14,400 PCMCIA V.17             Class-1 or Class-2

QuickCom      Spirit II                      Class-1

Supra         SupraFAXmodem V.32bis          Class-1 or Class-2

Silent Answer feature is supported.  The special command

+FAA=2 is required for operations as a Class 2 with Silent

Answer engaged, and +FEA=2 for Class 1 operations with Silent

Answer.

Supra         SupraFAXmodem V.32 bis Int.    Class-1

Supra         SupraFAXmodem V.32 bis LC      Class-1

Texas Instr.  TI4000E laptop modem           Class-1, Special command \J0\Q3

Toshiba       laptop Megahertz modems        Same as Megahertz P296FMV

TwinCom       TwinCom v1.13C                 Class-2

UDS           FasTalk Fax32                  Class-1

USRobotics    Courier HST Dual Std w/Fax     Cls-1,Spec cmd &H2,Recv EOP delay

USRobotics    Courier 14400 MicroChannel     Cls-1,Spec cmd &H2,Recv EOP delay

See below for special note.

USRobotics    Sportster 14,400 or 28,800     Cls-1,Spc cmd &H2&I2,RecvEOPdelay

If sending to Pitney Bowes machines, use: Spc cmd &H2&I2&R1S36=0

Do not use 'Check TCF signal' with this modem.

USRobotics    Courier 28800                  Cls-1,Spc cmd &H2&I2,RecvEOPdelay

USRobotics    V.terbo 21600 w/14400 Fax      Class-1, Recv EOP delay

USRobotics    Worldport 2496 V.42bis         SendFax

USRobotics    Worldport 9696 MNP5 FAX        SendFax

USRobotics    Worldport 9696 V.42 w/Fax      Class-1, Special command &H2

USRobotics    Worldport 14400 V.42 w/Fax     Class-1, Special command &H2

USRobotics    2400 PC / 2400 PC Fax          NOT COMPAT: SendFax non-spec

USRobotics    9600 Sportster                 NOT COMPAT: SendFax non-spec

VIVA          2496IF                         One model compatible

The VIVA board that is incompatible uses a slide switch

accessible from the rear of the computer to select the serial

port.  The VIVA board that is compatible uses on-card jumpers

to select the serial port setting.

ZyXEL         U-1496 (ROM 5.05 & 6.01)       Class-2 (ROM 6.00 is bad)

Zoltrix       ZX1896 or Zofax 96/24          SendFax

Zoltrix       9624 Class 2 (regular)         Class-2

Zoltrix       Class 2 with V.42 MNP          Class-2, might need 'No DLE esc'

Zoom          FaxModem FC 9624 (internal)    Class-2

Zoom          FaxModem FX 9624 (external)    Class-2

Zoom          V32bis (ROMs 1.56D,2.0 tested) Class-1 (or 2), Special cmd &C1

(ROM 1.6 bad)

SPECIAL NOTES:

CLASS 1 MODEM NOTE: For MOST Class 1 modems, using the 'Check TCF

signal' on the Modem Type page of the Settings notebook will provide

enhanced speed downgrading.  This may improve the quality of received

faxes.  On a few modems, engaging this switch will cause the modem

not to initialize properly.  Use this switch with care.  Never use

this switch with a US ROBOTICS SPORTSTER.

US ROBOTICS COURIER MicroChannel internal modem - has begun shipping

with a default setting loading during installation in the Reference

Diskette/ Partition that prevents return codes from reaching the fax

program.  THIS MUST BE CHANGED.  The user/installer must enter the

reference information area in the MicroChannel machine and ensure

that return code information is NOT blocked, otherwise the fax

program will NEVER be able to tell if your modem is responding

correctly, and will return an error message to you saying that it is

not responding correctly until you disable the return code blockage.

CONDENSED USER'S GUIDE

----------------------

This 32-bit fax software for OS/2 is fully enabled for the OS/2

Workplace Shell, taking advantage of the latest OS/2 user interface

standards.

It allows you to send and receive faxes of unlimited length and print

faxes to any OS/2 printer.  The FxPrint printer driver works with the

fax program for creating and sending faxes by "printing" from your

OS/2, DOS and Windows applications running on OS/2.  Features include

drag-and-drop printing and viewing, normal and fine fax resolution,

portrait and landscape orientation, multiple page lengths and printer

emulation for print capture (LPT3) from your DOS and Windows

applications.

If you have the Lite version and you send or receive faxes on a

regular basis, you will want to upgrade to the full retail version.

The full retail version has many additional features, including a

configurable toolbar, fax markup and editing, drawing, import,

export, automatic printing, gray-scale viewing for enhanced on-screen

readability, multiple phone books, enhanced log selection and

maintenance, and printer driver commands.  You can also get optical

character recognition (OCR), PCL, LAN versions with automatic

routing, multichannel support for up to 96 fax lines and programming

interfaces for developing your own "fax enabled" applications.

To purchase the full retail version or program options, see the

information on the program's Help Product Information screen.

READING THE QUICK REFERENCE GUIDE - A Quick Reference Guide page is

included in the program's fax log.  Run the program, open the fax

log, and double-click on the Reference Guide to view the page.  The

program can print any fax document to your OS/2 printer, so you can

print a copy of the Quick Reference Guide.

CONFIGURING THE PROGRAM - In the fax program's Settings notebook, set

the port used by your fax modem on the Ports page, your modem baud

rate and line type on the Modem page, and your fax modem values on

the Modem Type page.  See the program's README.DOC file for

recommended settings for your modem.  The Lite product supports fax

modems which follow the Class 1, Class 2 and SendFax standards.  The

full retail version includes support for many additional types of fax

hardware.

TO GET HELP - Check the message line as you move the mouse over a

toolbar button or menu item, or press F1 for extensive on-line help

information on any menu item that you have highlighted.  The Lite

version includes the same help system as the full retail product, so

some of the described features are not in the Lite version.

TRY THE TOOLBAR - The buttons in the toolbar provide "single-click"

access to the menu items.  When you put the mouse over a toolbar

button, the message line tells you about the button.  You can drop

fax file objects on some of the buttons to send, print or view the

fax.  In the full retail version, the toolbar is configurable using

drag-and-drop (drag the buttons, or drag menu commands to the toolbar).

USE POP-UP MENUS - As with other Workplace Shell operations, a

right-click of the mouse will provide a pop-up menu.  The contents of

the pop-up menu will vary based on what the mouse is on when you

press the right mouse button.  Different pop-up menus will appear if

you right-click on the main program window, the status window, the

fax log window, or the phone book window.

TAKE ADVANTAGE OF THE WORKPLACE SHELL FEATURES:

Drad-and-drop printing - If you use an OS/2 application that supports

printing of its document file objects by dropping them on printer

objects, then you can drop them on the FxPrint printer object to

create and send a fax.  Most modern OS/2 word processors now support

this feature.

More drag-and-drop - Double-click on a fax file object to view the

fax, or drag and drop a fax file object to do many useful things.

For example, you can drop it on any printer object to print it, on

the fax program object or into the fax workspace to view it, on the

Send toolbar button to send it, on the Print toolbar button to print

it, on any of the View or Orientation toolbar buttons to view it in

that size or orientation, or on the Log toolbar button to add a copy

of the fax to the fax log for later viewing or sending.

Resizable windows - Drag the borders of the program, log and phone

book windows to suite your preferences, and the program will remember

the sizes.

Fonts of your choice - Drag fonts from the OS/2 Font Palette and drop

them in many places, and the program will remember your font choices.

Drop them in the log, in the phone book, on the message line, or on

the status window.

PRINTING FAXES FROM OS/2 APPLICATIONS:

The FxPrint printer driver is a true OS/2 printer driver, and you use

it just like any other OS/2 printer driver.  The FxPrint printer

driver turns your print job into a fax document file, and an optional

Send pop-up dialog allows you to send the fax.

To create and send a fax from an OS/2 application, use the OS/2

application's Printer Setup command to select the FxPrint printer

object, then use the OS/2 application's Print command.

If you are using an OS/2 application that supports printing of its

document file objects by dropping them on a printer object, then you

can drop them on the FxPrint printer object to create and send a fax.

Most modern OS/2 word processors now support this feature.

You can configure the FxPrint printer driver for various page

lengths, orientations, and other settings.  The on-line help explains

the various options in detail.

Note that you can create several printer objects which use the

FxPrint printer driver, and each can have different "Job Properties"

in its settings notebook.

PRINTING FAXES FROM DOS APPLICATIONS:

The FxPrint printer driver provides emulation for the IBM Proprinter

X24 printer, so your DOS programs can create and send fax documents

by printing to the FxPrint printer object's LPT device.

By default, the FxPrint printer object is usually attached to LPT3.

FxPrint should be attached to an LPT device that is not used by any

other printer object.  You can check and set the LPT device on the

Output page of the FxPrint printer object's settings notebook.

To create and send faxes from DOS programs running on OS/2, have your

DOS program use LPT3 for printing (or whatever LPT device you have

set on the Output page of the FxPrint printer object).  If possible,

tell your DOS program that LPT3 is an IBM Proprinter X24 printer.

This will allow the DOS program to use the Proprinter's fonts and

graphics.  To produce a fax document that is easier to read, tell

your DOS program to use the Proprinter's "Elite PS" font or a bold

font.

If you want the results to be larger on the fax page, change the LPT

Emulation setting from "IBM Proprinter X24E" to "IBM PP X24E

Enlarged" by using the Job Properties button on the Printer driver

page of the FxPrint printer object's settings notebook.

If your DOS program can only produce high-quality printed output when

using a LaserJet printer, then you may want to upgrade to the full

retail version with the optional PCL5 emulation feature.  This allows

your DOS and Windows programs to print to the fax printer driver as

if it was a LaserJet III printer.

PRINTING FAXES FROM WINDOWS APPLICATIONS:

As described in the previous section, the FxPrint printer driver

provides emulation for the IBM Proprinter X24 printer.  Your Windows

programs can print to the Proprinter Windows printer driver, and that

printer driver can pass the data to the FxPrint printer object's LPT

device to create a fax document.

You must do a onetime installation/configuration of the Windows

printer driver.  To do this: 1) Run a full-screen Windows session,

such as by double-clicking on the WIN-OS/2 Full Screen object in the

Command Prompts folder.  2) Open the Windows Control Panel (in the

Main Group), then open Printers.  3) Press the Add button, select IBM

Proprinter X24, then press the Install button.  You will be prompted

to insert a printer driver diskette for installing the Proprinter

Windows printer driver.  If you are running OS/2 for Windows, this

will be one of your Windows installation diskettes.  If you are

running regular OS/2, this will be one of your OS/2 installation

diskettes.  4) Press the Connect button and set the printer driver's

device to the same LPT device used by the FxPrint printer driver

(like LPT3.OS2).  5) Installing and using the ATM (Adobe) fonts is

recommended (see your OS/2 documentation).

To create and send faxes from Windows programs running on OS/2, use

the application's Printer setup command to select the Proprinter

printer driver, then use the application's Print command.

If you want the results to be larger on the fax page, change the LPT

Emulation setting from "IBM Proprinter X24E" to "IBM PP X24E

Enlarged" by using the Job Properties button on the Printer driver

page of the FxPrint printer object's settings notebook.

If you upgrade to the full retail version with the optional PCL5

emulation feature, then your Windows programs print to the fax

printer driver as if it was a LaserJet III printer.

IF YOU HAVE PROBLEMS:

Most fax problems are due to improper modem settings and are easily

corrected.  Be sure to see the notes and suggestions about your fax

hardware at the beginning of this file.

Port selection - Be sure that the fax program is using the correct

port by checking the Ports page in the program's settings.  If your

fax hardware is turned on and attached to the specified port, they

you will hear it pick up the phone line and dial when you tell the

fax program to send a fax.  If it doesn't, try testing your fax modem

with a data communications program to make sure it is properly

attached and working.

Modem settings - Look at the Modem Type page in the program's

settings and check the settings for your fax hardware.  See the

README.DOC file for the recommended settings for your modem.  If your

modem is not listed, check your modem manual to determine if your

modem uses the Class 1, Class 2 or SendFax command set for faxing.

If your modem supports both Class 1 and Class 2 and you are having

problems in Class 2 mode, try changing it to Class 1.  As noted in

the modem information at the start of this file, some modems also

need additional check boxes or special commands on the Modem Type

page.

Finding the fax directory - If the fax program can't seem to find the

fax log, or if you have changed the location of your fax data

directory (which contains FAX.LOG, FAX.IDX, *.FAX and *.PBK files),

then you must tell the program where it is.  To do this, start the

fax program with the parameter of -L newdir where newdir is the full

pathname of the directory which contains the FAX.LOG file.  You only

need to do this once, and the fax program will then remember it.

Capturing details to track down problems - The fax program includes a

facility for capturing detailed information on its activities,

including all the commands and responses with your fax hardware.

Technical support will need this information to provide assistance.

To report a problem, first start the fax program with a parameter of

-V, recreate your problem, close the fax program, and fax or upload a

copy of the VOUT file (found in your fax data directory) to technical

support.

WANT MORE?  WITH THE FULL RETAIL VERSION, YOU GET:

Powerful fax editing, including the ability to sign your fax by

dropping your signature bitmap onto the fax, append other fax

documents by dropping them onto your fax, and type in any font by

dragging the font from the font palette.

Extended selection in the fax log so you can print or delete multiple

faxes with a single command, a Clean command to remove

"informational" log entries like "busy" or "no answer", and other log

maintenance and fax archiving features.

Multiple selection in the phone book, and support for multiple phone

books and distribution groups.

A gray-scale feature for improved viewing of faxes with smaller fonts

and images.

Options for automatically starting in receive mode and automatically

printing faxes when received or when sent.

A utility program for changing receive mode from the command line or

a command file (for example, to turn off receive mode so your data

program can use the fax modem), or to tell the program to view or

print a fax.

Options for OCR, PCL5 (LaserJet) emulation, LAN versions, multiline

versions, developer toolkits for doing fax broadcasting or

client-server programming, and more.

For information on upgrading to the full retail version or purchasing

program options, contact the publisher at the telephone numbers shown

on the program's Help Product Information screen.