Sega Master System and Game Gear Architecture

version 1.0 [09/30/1996]

by Marat Fayzullin [fms@freeflight.com]

WWW: http://www.freeflight.com/fms/

IRC: RST38h

The following document describes the hardware architecture of Sega

Master System, also known as Mark3 in the East (Korea, Japan). It also

covers the Sega GameGear, which is a later implementation of the same

architecture. Note that this document is in no way based on any official

Sega information and may be incomplete and incorrect in many places. "Sega

Master System" and "GameGear" are registered trademarks of Sega.

I would like to thank following people for their help in obtaining this

information (sorted alphabetically):

Richard Aplin

Neil Harding

Chris Shay

The current version of this file is missing some information. I will add

these parts in later releases. If you have any information on SMS/GG,

which is not in this manual, feel free to write to fms@freeflight.com.

Your help will be appreciated.

******************************* Contents *******************************

1. General Architecture

2. I/O Ports

3. Joysticks

4. Nationalization

5. VDP

a) VRAM

b) Palette Registers

c) Mode Registers

d) Status Register

e) Screen

f) Sprites

6. Memory Mapper

7. Cartridge Pinouts

************************* General Architecture *************************

SMS contains following main parts:

* Zilog Z80 CPU at about 3.3MHz (53.203Mhz/16).

* Custom video controller (VDP) based on a well known TMS9918/9928

chip from Texas Instruments, but not equal to it. This chip

provides 256x192x16 tiled screen and 64 16-color sprites. The

VRAM is separated from main RAM and accessed via VDP ports.

* Generic SN76489 PSG made by Texas Instruments. It has 3 tone

channels and a noise channel. The clock frequency of this chip

appears to be around 4194304Hz. Refer to SN76489 docs for more

information.

Input/output is implemented via standard Z80 I/O ports, except for the

memory mapper registers (see "Memory Mapper").

The Maskable Interrupt (with vector at 0038h) is generated by a VDP chip

on each vertical blanking impulse (VBlank). There is also a Non-Maskable

Interrupt (with vector at 0066h) which occurs when the user presses the

PAUSE button on the SMS console. GameGear has no PAUSE button and uses

non-maskable interrupts to acknowledge serial I/O.

GG has somewhat upgraded VDP and some changes in the joystick interface

and memory mapping, but the rest of its hardware seems to be identical to

SMS.

Following is a memory map of SMS/GG:

-- 10000 ----------------------------------------------

Mirror of RAM at C000-DFFF

--- E000 ----------------------------------------------

8kB of on-board RAM (mirrored at E000-FFFF)

--- C000 ----------------------------------------------

16kB ROM Page #2, Cartridge RAM

--- 8000 ----------------------------------------------

16kB ROM Page #1

--- 4000 ----------------------------------------------

16kB ROM Page #0

--- 0000 ----------------------------------------------

Locations FFFCh-FFFFh and DFFCh-DFFFh belong to the Memory Mapper (see

"Memory Mapper").

****************************** I/O ports *******************************

Following is an I/O map of SMS/GG:

-- DC -- JOYPAD1 ---------------------------------------------- READ-ONLY --

This is one of two joystick ports. Each bit corresponds to a button: 1

for pressed, 0 for released. This port is also mirrored at address C0h

(see Pit-Pot cartridge) and probably at some other officially unused

addresses. Its bits are laid out as follows:

bit   MasterSystem                    GameGear

7     Joypad #2 DOWN                  Not used

6     Joypad #2 UP                    Not used

5     Joypad #1 FIRE-B                Joypad FIRE-B

4     Joypad #1 FIRE-A                Joypad FIRE-A

3     Joypad #1 RIGHT                 Joypad RIGHT

2     Joypad #1 LEFT                  Joypad LEFT

1     Joypad #1 DOWN                  Joypad DOWN

0     Joypad #1 UP                    Joypad UP

-- DD -- JOYPAD2 ---------------------------------- SMS-ONLY -- READ-ONLY --

This is the second of two joystick ports. Each bit corresponds to a

button: 1 for pressed, 0 for released. This port is also mirrored at

address C1h (see Pit-Pot cartridge) and probably at some other officially

unused addresses. Its bits are laid out as

bit   MasterSystem

7     Lightgun #2

6     Lightgun #1

5     Not used

4     RESET button

3     Joypad #2 FIRE-B

2     Joypad #2 FIRE-A

1     Joypad #2 RIGHT

0     Joypad #2 LEFT

This port doesn't appear to be present in GG, as GG has no second

joystick and no lightgun. The upper two bits (7 and 6) are also used for

automatic nationalization (see "Nationalization").

-- 00 -- JOYPAD3 ----------------------------------- GG-ONLY -- READ-ONLY --

This port is only present in the GG and contains the START button bit

and the nationalization bit:

bit   GameGear

7     START button (1 when pressed, 0 when released)

6     Nationalization (1 in European/American GG, 0 in Japanese GG)

-- 7E -- CURLINE/PSGPORT ------------------------------------- READ/WRITE --

This port is performing a dual role. When read, it contains the current

scanline number (CURLINE). When written, it serves to output data into the

PSG chip (PSGPORT). The 7Eh port also appears to be mirrored at address

7Fh and probably at some other officially unused addresses.

-- BE -- VDPDATA --------------------------------------------- READ/WRITE --

This is a VDP chip port used to read and write data from/to VRAM. See

"VDP" chapter for more information.

-- BF -- VDPSTAT/VDPCTRL ------------------------------------- READ/WRITE --

When read, this port returns the VDP status word as follows:

bit   Function

7     VBlank Flag, sets to 1 at the beginning of each Vertical Blanking

Impulse. This flag resets to 0 after the register is read.

6

5

When written, this port is used to pass VRAM addresses, register

numbers, and color numbers to VDP. See "VDP" chapter for more information.

The BFh port is also mirrored at address BDh (see SailorMoon cartridge)

and probably at some other officially unused addresses.

-- 3F -- GUNPORT --------------------------------- SMS-ONLY -- WRITE-ONLY --

This port is only present in the SMS and serves both for lightgun

initialization and for determining the system nationalization in SMS.

*************************** Nationalization ****************************

Both SMS and GG contain circuitry allowing to determine whether the

program is running on Japanese or European/American hardware.

In SMS, the way to find the hardware nationalization is somewhat tricky.

A program is supposed to write ???

In the case of GG, the hardware type can be figured by analyzing the 6th

bit of the 00h port. This bit is 1 for European/American GG and 0 for

Japanese GG.

The GG cartridges also have a data byte which contains information on

the ROM size and the market for which cartridge has been manufactured.

This byte is located at address 7FFFh (3FFFh in page #1 for mapped

cartridges) and its bits have the following meaning:

bit   Function

7-4   Country Code

0101  Japan

0110  Japan, USA, Europe

0111  USA, Europe

3-0   ROM Size

0000  >128kB

1100  32kB

1101  64kB

1110  128kB

********************************* VDP **********************************

The VDP used in SMS is a heavily modified version of TMS9918/9928 chips

made by Texas Instruments and used in such machines as MSX and TI99/4. In

the SMS version, all screen modes are gone and replaced by a single tiled

mode. Nevertheless, the VDP still has Control and Data ports, as well as

Mode Registers and a Status Register. The sprite handling is also changed,

and a programmable palette is added for graphics and sprites. The palette

contains 32 colors out of 64, 16 for graphics + 16 for sprites. The screen

size is 256x192, but may reach 264x192 in the overscan mode.

The GG uses the same VDP as SMS, but some CRT-dependent functions (such

as overscan) are gone from it, and the palette can be selected from 512

colors instead of 64. The GG screen is actually a 160x144 "window" in the

middle of a standard SMS screen.

a) VRAM

The VideoRAM (VRAM) is a 16kB chunk of memory where VDP stores screen

graphics and sprites. The CPU can not access VRAM directly and has to

use VDP ports instead. Here is how:

Writing to VRAM (address in HL, data written from D)

-----------------------------------------------------------------------

LD A,L         ; Output lower byte of address into VDPCTRL (port BFh).

OUTA (BFh)     ;

LD A,H         ; Output higher byte of address into VDPCTRL. The upper

OUTA (BFh)     ; two bits must be 00.

LD A,C         ; Output data into VDPDATA (port BEh). The address will

OUTA (BEh)     ; be autoincremented after each write.

...

-----------------------------------------------------------------------

Reading from VRAM (address in HL, data read into A)

-----------------------------------------------------------------------

LD A,L         ; Output lower byte of address into VDPCTRL (port BFh).

OUTA (BFh)     ;

LD A,H         ; Output higher byte of address into VDPCTRL. The upper

OUTA (BFh)     ; two bits must be 00.

LD A,C         ; Input data from VDPDATA (port BEh). The address will

INA (BEh)      ; be autoincremented after each read.

...

-----------------------------------------------------------------------

b) Palette Registers

There are 32 palette registers. First 16 of them are used for picture

colors, the remaining 16 are used for sprites. As GG offers a bigger

selection of colors, its palette registers are 2 bytes each, while SMS

registers are 1 byte. Here is how to access the palette:

Setting SMS palette (address 00h-1Fh in C, 00BBGGRR value in D)

-----------------------------------------------------------------------

LD A,C         ; Output the palette address into VDPCTRL (port BFh).

LD A,C0h       ; Output C0h into VDPCTRL. The upper two bits must

OUTA (BFh)     ; be 11.

LD A,D         ; Output values into VDPDATA (port BEh). The register

OUTA (BEh)     ; number will be autoincremented after each write.

...

-----------------------------------------------------------------------

Setting GG palette (address 00h-3Fh in C, 0000BBB0GGG0RRR0 value in DE)

-----------------------------------------------------------------------

LD A,C         ; Output the palette address into VDPCTRL (port BFh).

LD A,C0h       ; Output C0h into VDPCTRL. The upper two bits must

OUTA (BFh)     ; be 11.

LD A,E         ; Output values into VDPDATA (port BEh), lower byte

OUTA (BEh)     ; first, upper byte second. The register number will be

LD A,D         ; autoincremented after each write.

OUTA (BEh)     ;

...

-----------------------------------------------------------------------

c) Mode Registers

The VDP has 16 internal registers which control different modes and

addresses of graphics and sprites in VRAM. The registers are write-only

and accessed as follows:

Writing to VDP mode register (register number in C, data in D)

-----------------------------------------------------------------------

LD A,D         ; Output data into VDPCTRL (port BFh).

OUTA (BFh)     ;

LD A,E         ; Output register number ORed with 80h into VDPCTRL.

OR A,80h       ; The upper four bits must be 10.

OUTA (BFh)     ;

-----------------------------------------------------------------------

-- VDP[0] ------------------------------------------------------------------

General mode register. Its bits are laid out in a following way:

bit   Function (when bit set to 1)

7     Do not scroll right 2 columns (16 pixels) of the screen.

6     Do not scroll top 2 rows (16 pixels) of the screen.

5     Do not show leftmost column (8 pixels) of the screen.

4     Generate line interrupts (see VDP[???]).

3     Shift all sprites 8 pixels to the left.

2     Unknown

1     Enable overscan (SMS only). Makes screen 264 pixels wide.

0     Unknown

-- VDP[1] ------------------------------------------------------------------

General mode register. Its bits are laid out in a following way:

bit   Function (when bit set to 1)

7     Unused (?)

6     Enable display. No picture is shown when this bit = 0.

5     Generate vertical (VBlank) interrupts.

4

3

2     Unused (?)

1     8x16 sprites. When this bit set to 0, sprites are 8x8.

0     Double all sprite pixels in size (zoomed sprites).

**************************** Memory Mapper *****************************

************************** Cartridge Pinouts ***************************

The following data has been provided by