WESTERN DIGITAL PLUS AND ELITE

FAMILY LAN ADAPTER DRIVERS

FOR USE IN NDIS ENVIRONMENTS

INSTALLATION GUIDE

------------------

This installation guide provides the following instructions:

1.0 Overview

2.0 DOS Instructions

2.1 Installing LAN Manager on DOS

2.2 Installing 3Com 3+Open version 1.1 on DOS

2.3 Installing IBM LAN Requestor on DOS

2.3.1 Installing IBM PC LAN Support

2.3.2 Installing IBM DOS LAN Requestor

2.4 Installing IBM PC LAN on DOS

2.4.1 Installing IBM LAN Support

2.4.2 Installing IBM PC LAN After LAN Support

2.5 Installing Other Programs on DOS

3.0 OS/2 Instructions

3.1 Installing LAN Manager on OS/2

3.2 Installing 3Com 3+Open version 1.1 on OS/2

3.3 Installing IBM LAN Server on OS/2

4.0 Installing Two Adapters in the Same System

5.0 PROTOCOL.INI Parameters

6.0 Error Messages

1.0	OVERVIEW

The NDIS driver is designed to work with both the Western Digital

"PLUS" and "ELITE" (WD8003/WD8013) families of LAN Adapters.

This driver conforms to the Network Driver Interface Specification

(NDIS) defined by Microsoft/3Com. The driver has been tested with

Microsoft LAN Manager, 3Com 3+Open, IBM LAN Server, IBM PC LAN,

and other NDIS compatible network operating systems and protocol

stacks.

The driver provides support for these three adapter types:

7  Jumper-Configured

These adapters can only be configured by setting jumpers

on the adapter boards.

WD8013EBT   WD8003E

WD8003EBT   WD8003S

WD8003SH    WD8003WT

7 Software-Configurable

These adapter cards have the ability to be configured using

a software setup program.

WD8003EP    WD8013EP

WD8013EW    WD8013W

WD8003EB    WD8003W

7 Micro Channel

This adapter type works only with the IBM Micro Channel bus

protocol standard.

WD8003ET/A  WD8003E/A

WD8003W/A   WD8003ST/A

WD8013WP/A  WD8013WP/A

These files are included with this release of the WD NDIS driver

and are located on the SuperDisk shipped with your Western

Digital adapter board:

INSTALL.DOC           Installation Guide (this File)

RELEASE.DOC           Version history and release information

MACWD.OS2             OS/2 NDIS MAC driver

MACWD.DOS             DOS NDIS MAC driver

MACWD2.OS2            OS/2 NDIS MAC driver for second LAN adapter

MACWD2.DOS            DOS NDIS MAC driver for second LAN adapter

PROTOCOL.INI          PROTOCOL.INI file sample

Install the adapter card in your PC using the instructions in the

Installation Guide included with the adapter.  Proper installation

can be verified by running the Diagnostic Program under DOS.

NOTE

The Diagnostic Program will not run properly from

the OS/2 compatibility box.

2.0    DOS INSTRUCTIONS

The following section provides you with instructions for installation

all drivers based on the MS-DOS platform. This includes instructions

on modifying the CONFIG.SYS and PROTOCOL.INI files to configure the

specific driver.

2.1    INSTALLING LAN MANAGER ON DOS

The following instructions only apply to installing the LAN Manager

on a DOS platform. Do not start LAN Manager until after it's

installed. Use the Microsoft instructions to complete the instal-

lation procedure. Microsoft includes support for the Western Digital

PLUS family adapters.  Select the Western Digital adapter from the

list provided by Microsoft.

To install the LAN Manager on a DOS platform:

1. Copy the LAN Manager Driver to the driver directory used by LAN

Manager.  For example, when installing a LAN Manager server or

enhanced workstation using the normal directory structure type:

COPY MACWD.DOS C:\LANMAN\DRIVERS\ETHERNET

When installing a LAN Manager basic workstation type:

COPY MACWD.DOS C:\LANMAN

2. Modify the PROTOCOL.INI in accordance with the adapter type.

a. For jumper-configured adapters, use a text editor to add the

following lines to the file:

[MACWD_NIF]

DriverName = MACWD$

IOBase = 0x280

RAMAddress = 0xD000

IRQ = 3

Replace the above IOBase, RAMAddress, and IRQ values with

the values appropriate for your adapter card. If you aren't

sure which adapter type you have, refer to Section 1.0 or

use the setup above. A more complete explanation of these

parameters and their appropriate values are provided in

section 5.0. If you have a software-configurable or Micro

Channel adapter, you can simplify your parameters by using

step 2b.

b. For software-configurable and Micro Channel adapters, use a

text editor to add the following lines to the file:

[MACWD_NIF]

DriverName = MACWD$

3. Reboot the system and bring up LAN Manager using the instructions

supplied by the LAN Manager vendor.

2.2    INSTALLING 3+OPEN VERSION 1.1 ON DOS

Install 3+Open using 3Com instructions.  3Com does not include

support for Western Digital adapters, therefore select a similar

adapter such as 3Com EtherLink II during installation.

To install 3+Open on DOS, follow these instructions:

1.  Copy the MACWD.DOS Driver from this diskette to the driver

directory used by 3+Open.

For example, when installing a 3+Open DOS workstation using

the normal directory structure type:

COPY MACWD.DOS C:\3OPEN\DOSWKSTA\LANMAN\DRIVERS

2. Modify CONFIG.SYS file to reference this adapter rather than

the adapter selected during 3+Open installation.

For example, if the 3Com EtherLink II was selected during

a workstation installation, use a text editor to change

the line:

DEVICE=\3OPEN\DOSWKSTA\LANMAN\DRIVERS\ELNNKII.DOS

to the line:

DEVICE=\3OPEN\DOSWKSTA\LANMAN\DRIVERS\MACWD.DOS

NOTE

Other varieties of LAN Manager keep their drivers

in different directories. The objective is to have

the CONFIG.SYS reference the MACWD.DOS driver rather

than the adapter driver selected during installation.

Also the LAN Manager installation adds several drivers

to the CONFIG.SYS file. Only the MAC driver referring

to a particular board should be replaced. Other drivers,

such as PROTMAN.DOS should not be modified.

3. Modify PROTOCOL.INI in accordance with your adapter type.

a. For jumper-configured adapters, use a text editor to add

the following lines to the file:

[WDNDIS]

DriverName = MACWD$

IOBase = 0x280

RAMAddress = 0xD000

IRQ = 3

Replace the above IOBase, RAMAddress, and IRQ values with

the values appropriate for your adapter card. If you aren't

sure which adapter type you have, refer to Section 1.0 or

use the setup above. A more complete explanation of these

parameters and their appropriate values are provided in

section 5.0. If you have a software-configurable or Micro

Channel adapter, you can simplify your parameters by using

step 3b.

The '[WDNDIS]' statement is used in the Bindings command

to associate the protocol stack with the driver.

b.  For software-configurable and Micro Channel adapters, use a

text editor to add the following lines to the file:

[WDNDIS]

DriverName = MACWD$

The '[WDNDIS]' statement is used in the Bindings command

to associate the protocol stack with the driver.

4. Locate the section of the PROTOCOL.INI file which describes the

protocol stack for your configuration. Locate the 'BINDINGS ='

statement within this section and use a text editor to modify this

statement to reference 'WDNDIS' rather than the name set during

installation.

For example, under 3+Open with the DLC/NETBEUI protocol stack and

3Com EtherLink II selected during installation, in the PROTOCOL.INI

file section labeled '[DLC]', the lines:

[DLC]

DriverName = DLC$

BINDINGS = EtherlinkII

must be changed to:

[DLC]

DriverName = DLC$

BINDINGS = WDNDIS

This changes the bindings of the the protocol stack.

5. Reboot the system and bring up 3+Open using the instructions

supplied by the 3Com.

2.3    INSTALLING IBM LAN REQUESTOR ON DOS

IBM LAN Server must run in conjunction with IBM PC LAN Support

first Program to work with a NDIS driver. Install IBM PC LAN

Support then IBM LAN Server.

2.3.1  Installing IBM PC LAN Support Program

Here is the procedure for installing IBM LAN Support:

1. Copy the MACWD.DOS file to the 'C:\LANMAN' subdirectory.

COPY MACWD.DOS C:\LANMAN

2. Install IBM LAN as the manual explains. Select 'Ethernet' as

the network type.

3. When asked by the install program whether the line DXMT0MOD.SYS is

appropriate to add to the CONFIG.SYS file, edit the line to read:

DEVICE=DXMT0MOD.SYS O=N

This tells the IBM LAN SUPPORT program not to reset the card when it

is starting.

4. Change the line in the CONFIG.SYS file which reads

DEVICE=XXX.DOS

to read

DEVICE=C:\LANMAN\MACWD.DOS

5. Modify PROTOCOL.INI in accordance with your adapter type.

a. For jumper-configured adapters, use a text editor to add

the following lines to the file:

[WDMAC]

DriverName = MACWD$

IOBase = 0x280

RAMAddress = 0xD000

IRQ = 3

Replace the above IOBase, RAMAddress, and IRQ values with

the values appropriate for your adapter card. If you aren't

sure which adapter type you have, refer to Section 1.0 or

use the setup above. A more complete explanation of these

parameters and their appropriate values are provided in

section 5.0. If you have a software-configurable or Micro

Channel adapter, you can simplify your parameters by using

step 5b.

b. For software-configurable and Micro Channel adapters, use a

text editor to add the following lines to the file:

[WDMAC]

DriverName = MACWD$

6. Reset the computer to start IBM LAN SUPPORT.

2.3.2  Installing IBM DOS LAN Requester

Here is the procedure for installing the IBM DOS LAN Requester:

1. Install, but do not start, DOS LAN Requester according to the

instructions provided with DOS LAN Requester.

2. Change the line in the CONFIG.SYS file that contains

DEVICE=DXMT0MOD.SYS S=12 C=14 ST=12

because the parameters might not be the same. Edit the line

to read:

DEVICE=DXMT0MOD.SYS S=12 C=14 ST=12 O=N

for the Western Digital adapter.

3. Modify PROTOCOL.INI in accordance with your adapter type.

a. For jumper-configured adapters, use a text editor to add

the following lines to the file:

[WDMAC]

DriverName = MACWD$

IOBase = 0x280

RAMAddress = 0xD000

IRQ = 3

Replace the above IOBase, RAMAddress, and IRQ values with

the values appropriate for your adapter card. If you aren't

sure which adapter type you have, refer to Section 1.0 or

use the setup above. A more complete explanation of these

parameters and their appropriate values are provided in

section 5.0. If you have a software-configurable or Micro

Channel adapter, you can simplify your parameters by using

step 3b.

b. For software-configurable and Micro Channel adapters, use a

text editor to add the following lines to the file:

[WDMAC]

DriverName = MACWD$

2.4    INSTALLING IBM PC LAN ON DOS

IBM PC LAN must run in conjunction with IBM PC LAN Support to work

with a NDIS driver. Install IBM PC LAN Support first, then IBM PC

LAN.

2.4.1  Installing IBM PC LAN Support

To install IBM PC LAN Support on your system, follow this procedure:

1. Copy the MACWD.DOS file to the 'C:\LANMAN' subdirectory.

COPY MACWD.DOS C:\LANMAN

2. Install IBM PC LAN Support as described in the manual. Choose

'Etherand' as the network type.

3. When asked by the install program whether the line DXMT0MOD.SYS is

appropriate to add to the CONFIG.SYS file, edit the line to read:

DEVICE=DXMT0MOD.SYS O=N

This tells the IBM PC LAN Support Program not to reset the card

when it is starting.

4. Change the line in the CONFIG.SYS file which reads:

DEVICE=XXX.DOS

to read

DEVICE=C:\LANMAN\MACWD.DOS

5. Modify PROTOCOL.INI in accordance with your adapter type.

a. For jumper-configured adapters, use a text editor to add

the following lines to the file:

[WDMAC]

DriverName = MACWD$

IOBase = 0x280

RAMAddress = 0xD000

IRQ = 3

Replace the above IOBase, RAMAddress, and IRQ values with

the values appropriate for your adapter card. If you aren't

sure which adapter type you have, refer to Section 1.0 or

use the setup above. A more complete explanation of these

parameters and their appropriate values are provided in

section 5.0. If you have a software-configurable or Micro

Channel adapter, you can simplify your parameters by using

step 5b.

b. For software-configurable and Micro Channel adapters, use a

text editor to add the following lines to the file:

[WDMAC]

DriverName = MACWD$

6. Reset the computer to start IBM LAN SUPPORT.

2.4.2  Installing IBM PC LAN After LAN Support Installation

To install the IBM PC LAN after installing the LAN Support, follow

this procedure:

1. Install IBM PC LAN as described in the manual.  Choose the WESTERN

DIGITAL card option.

2. When the installation is complete, verify that the PROTOCOL.INI

file contains the correct bindings.  The file is located in the

'C:\LANMAN' subdirectory.

2.5    Installing Other Systems on DOS

There are several other DOS environments capable of supporting the

NDIS driver. These include: DEC's DECnet, Wollongong's WIN TCPIP,

Banyan's Vines Workstation environment, Sun's PCNFS Workstation

environment, NCSA's Telnet, and  FTP's PCTCP. These DOS environments

should handle the task of installation in much the same manner.

1. Copy the MACWD.DOS file to the appropriate subdirectory. For

example:

COPY MACWD.DOS C:\LANMAN

For information on which directory you must use to install the

driver, refer to the relevant manual.

2. Modify the CONFIG.SYS file by adding this line:

DEVICE=C:\LAN\MACWD.DOS

Where the 'LAN' subdirectory is the location of the driver.

3. Modify PROTOCOL.INI in accordance with your adapter type.

a. For jumper-configured adapters, use a text editor to add

the following lines to the file:

[WDMAC]

DriverName = MACWD$

IOBase = 0x280

RAMAddress = 0xD000

IRQ = 3

Replace the above IOBase, RAMAddress, and IRQ values

with values appropriate for your adapter card. A more

complete explanation of these parameters and their

appropriate values are provided in section 5.0. If you

have a software-configurable or Micro Channel adapter,

you can simplify your parameters by using step 3b.

The '[WDMAC]' statement is used in the Bindings command

to associate the protocol stack with the driver.

b. For software-configurable and Micro Channel adapters, use a

text editor to add the following lines to the file:

[WDMAC]

DriverName = MACWD$

The '[WDMAC]' statement is used in the Bindings command

to associate the protocol stack with the driver.

4. Locate the section of the PROTOCOL.INI file which describes the

protocol stack for your configuration. Locate the 'BINDINGS ='

statement within this section and use a text editor to modify this

statement to reference 'WDMAC' rather than the name set during

installation.

5. Reboot the computer.

3.0    OS/2 INSTRUCTIONS

The following section provides you with instructions for installation

all drivers based on the OS/2 platform. This includes instructions on

modifying the CONFIG.SYS and PROTOCOL.INI files to configure the

specific driver.

3.1    INSTALLING LAN MANAGER ON OS/2

Install, but do not start LAN Manager, using the Microsoft

instructions.  Microsoft includes support for the Western Digital

PLUS family adapters. Select the Western Digital adapter from the

list provided by Microsoft.

To install the LAN Manager on OS/2, follow this procedure:

1. Copy the newest LAN Manager driver to your hard drive. Copy the

OS/2 driver from this diskette to the driver directory used by LAN

Manager.

For example, when using the normal directory structure type:

COPY MACWD.OS2 C:\LANMAN\DRIVERS\ETHERNET

2. Modify PROTOCOL.INI in accordance with your adapter type.

a. For jumper-configured adapters, use a text editor to add

the following lines to the file:

[MACWD_NIF]

DriverName = MACWD$

IOBase = 0x280

RAMAddress = 0xD000

IRQ = 3

Replace the above IOBase, RAMAddress, and IRQ values with

the values appropriate for your adapter card. If you aren't

sure which adapter type you have, refer to Section 1.0 or

use the setup above. A more complete explanation of these

parameters and their appropriate values are provided in

section 5.0. If you have a software-configurable or Micro

Channel adapter, you can simplify your parameters by using

step 2b.

b. For software-configurable and Micro Channel adapters, use a

text editor to add the following lines to the file:

[MACWD_NIF]

DriverName = MACWD$

3. Reboot and bring up LAN Manager using the instructions supplied by

the LAN Manager vendor.

3.2    INSTALLING 3+OPEN VERSION 1.1 ON OS/2

Install 3+Open using 3Com instructions. 3Com does not include

support for Western Digital adapters, therefore select a

similar adapter such as 3Com EtherLink II during installation.

To install 3+Open on OS/2, follow this procedure:

1. Copy the OS/2 driver from this diskette to the driver directory

used by 3+OPEN.

For example, when installing a 3+Open server using the normal

directory structure type:

COPY MACWD.OS2 C:\3OPEN\SERVER\LANMAN\DRIVERS

Or, when installing a 3+Open OS/2 workstation type:

COPY MACWD.OS2 C:\3OPEN\OS2WKSTA\LANMAN\DRIVERS

2. Modify the CONFIG.SYS or CONFIG.OS2 file to reference this adapter

rather than the adapter selected during 3+Open installation.

For example, if the 3Com EtherLink II was selected during a server

installation, use a text editor to change the line:

DEVICE=\3OPEN\SERVER\LANMAN\DRIVERS\ELNNKII.OS2

to the line:

DEVICE=\3OPEN\SERVER\LANMAN\DRIVERS\MACWD.OS2

If the 3Com EtherLink II was selected during a workstation

installation, use a text editor to change the line:

DEVICE=\3OPEN\OS2WKSTA\LANMAN\DRIVERS\ELNNKII.OS2

to the line:

DEVICE=\3OPEN\OS2WKSTA\LANMAN\DRIVERS\MACWD.OS2

NOTE

Other varieties of LAN Manager keep their

drivers in different directories.  The

objective is to have the CONFIG.SYS reference

the MACWD.OS2 driver rather than the adapter

driver selected during installation.  Also

that the 3+Open installation adds several

drivers to the CONFIG.SYS file.  Only the MAC

driver referring to a particular board should

be replaced. Other drivers, such as PROTMAN.OS2

should not be modified.

5. Modify PROTOCOL.INI in accordance with your adapter type.

a. For jumper-configured adapters, use a text editor to add

the following lines to the file:

[WDNDIS]

DriverName = MACWD$

IOBase = 0x280

RAMAddress = 0xD000

IRQ = 3

Replace the above IOBase, RAMAddress, and IRQ values with

the values appropriate for your adapter card. If you aren't

sure which adapter type you have, refer to Section 1.0 or

use the setup above. A more complete explanation of these

parameters and their appropriate values are provided in

section 5.0. If you have a software-configurable or Micro

Channel adapter, you can simplify your parameters by using

step 5b.

The '[WDNDIS]' statement is used in the Bindings command to

associate the protocol stack with the driver.

b.  For software-configurable and Micro Channel adapters, use a

text editor to add the following lines to the file:

[WDNDIS]

DriverName = MACWD$

The '[WDNDIS]' statement is used in the Bindings command

to associate the protocol stack with the driver.

6. Locate the section of the PROTOCOL.INI file which describes the

protocol stack for your configuration.  Locate the 'BINDINGS ='

statement within this section and use a text editor to modify this

statement to reference 'WDNDIS' rather than the name set during

installation.

For example, under 3+Open with the XNS protocol stack and 3Com

EtherLink II selected during installation, in the PROTOCOL.INI file

section labeled '[XNS @_NETBIOS]', the line:

[DLC]

DriverName = DLC$

BINDINGS = ETHERLINKII

must be changed to:

[DLC]

DriverName = DLC$

BINDINGS = WD8003XMAC

If the DLC/NETBEUI protocol stack was selected for use with

3+Open, the same lines must be changed in the PROTOCOL.INI file

section labeled '[DLC]'.

This changes the bindings of the the protocol stack.

7. Reboot the system and bring up 3+Open using the instructions

supplied by the 3Com.

3.3   INSTALLING IBM LAN SERVER ON OS/2

This package has built-in support for the Western Digital Ethernet

series of cards. Install LAN SERVER as the IBM manual describes,

choosing the Western Digital card option that appears on the

screen.  Do not start LAN SERVER yet.

To install the IBM LAN Server on OS/2, follow this procedure:

1. Copy the latest version of the driver over the driver included by

IBM, like this:

COPY MACWD.OS2 C:\CMLIB

2. Modify PROTOCOL.INI in accordance with your adapter type.

a. For jumper-configured adapters, use a text editor to add

the following lines to the file:

[WDMAC]

DriverName = MACWD$

IOBase = 0x280

RAMAddress = 0xD000

IRQ = 3

Replace the above IOBase, RAMAddress, and IRQ values with

values appropriate for your adapter card. If you aren't

sure which adapter type you have, refer to Section 1.0 or

use the setup above. A more complete explanation of these

parameters and their appropriate values are provided in

section 5.0. If you have a software-configurable or Micro

Channel adapter, you can simplify your parameters by using

step 2b.

b. For software-configurable and Micro Channel adapters, use a

text editor to add the following lines to the file:

[WDMAC]

DriverName = MACWD$

3. Reboot and bring up LAN SERVER using the instruction supplied

by IBM.

4.0   INSTALLING TWO ADAPTERS IN THE SAME SYSTEM

This release includes MACWD2.OS2 and MACWD2.DOS.  These drivers may

be used to install a second adapter card.  When installing the LAN

Manager software, select two Western Digital family adapters.

When LAN Manager setup exits, modify the CONFIG.SYS file and the

PROTOCOL.INI file using this procedure.

1. Modify CONFIG.SYS by changing one of the two MACWD.XXX entries

from MACWD.XXX to MACWD2.XXX.  For example:

DEVICE=C:\LANMAN\DRIVERS\ETHERNET\MACWD\MACWD.DOS

DEVICE=C:\LANMAN\DRIVERS\ETHERNET\MACWD\MACWD.DOS

should be changed to:

DEVICE=C:\LANMAN\DRIVERS\ETHERNET\MACWD\MACWD.DOS

DEVICE=C:\LANMAN\DRIVERS\ETHERNET\MACWD\MACWD2.DOS

2. Modify PROTOCOL.INI by changing the second adapter entry to

specify the DriverName as MACWD2$ and changing all parameter

values so no conflict results.

a. For jumper-configured adapters, use a text editor to

modify the following lines for the second adapter entry:

[MACWD2_NIF]

DriverName = MACWD2$

IOBase = 0x300

RAMAddress = 0xD800

IRQ = 5

Make sure these parameter values do not conflict with

the first adapter entry, and that they match the settings

of the second adapter.

b. For software-configurable adapters, use a text editor to

modify the following lines for the second adapter entry:

[MACWD2_NIF]

DriverName = MACWD2$

IOBase = 0x300

Make sure the IOBase value does not conflict with the

value provided for the first adapter, and that it matches

the second adapter setting. Always include an IOBase

statement with the DriverName.

c. For Micro Channel adapters, use a text editor to modify

the following lines for the second adapter entry:

[MACWD2_NIF]

DriverName = MACWD2$

SlotNumber = 4

Make sure the SlotNumber value does not conflict with the

value provided for the first adapter, and that it matches

the second adapter setting. Always include a SlotNumber

statement with the DriverName.

3. Modify adapter values so that the two adapters do not use the same

values for I/O Base Address, RAM Address and IRQ.

Refer to the adapter's Installation Guide for information about

these parameters.

4. Load the drivers and reboot the system.

5.0   PROTOCOL.INI PARAMETERS

Several parameters are typically specified in the MACWD section of

the PROTOCOL.INI file. The only parameter that must be specified

is the 'DriverName'. All others have default values supplied

by the driver and can be left out of the PROTOCOL.INI file if the

installed adapter matches the default configuration. Software-

configurable and Micro Channel adapters ignore the values of IRQ,

IOBase and RAMAddress and instead use the values in the POS

registers set up when the card was installed.

Below is a list of parameters and their definitions commonly found

in the PROTOCOL.INI file.  Both default values and possible value

ranges are provided for each parameter.

IRQ     Specifies the interrupt used by the adapter.  The

default is 3.  IRQ3 is used by COM2.  If COM2 is in

use, choose another IRQ value.  The interrupt is

selected via a jumper on some adapters, or by way of

the EZSETUP program supplied with other adapters.

Consult the installation guide for information on how

to determine or modify the interrupt. This parameter

is always a decimal number in the range of 2 - 15.

Not all values are supported by all adapters.

IOBase     Specifies the base I/O address used by the adapter.

The default is 0x280.  The address is selected via a

jumper on some adapters, or by way of the EZSETUP

program supplied with other adapters.  Consult the

Installation Guide for information on how to determine

or modify the address.  This parameter is a hexadecimal

number in the range of 0x200 - 0xFFFF.  Hexadecimal

numbers must be preceded by '0x' to distinguish them

from decimal numbers.  Remeber that not all values are

supported by all adapters.

RAMAddress     Specifies the Shared RAM segment address used by the

adapter.  This value is established under software

control by the driver.  The default Shared RAM segment

address is 0xD000.  This address is recommended unless

it conflicts with some other device in the PC.  Consult

the adapter Installation Guide or print the file

SYSMAP.DOC on the SuperDisk for more information on how

to select a shared RAM address.  The driver uses

hexadecimal values in the range of 0xA000 - 0xFFFF.

SlotNumber     Specifies the number of the slot containing the adapter

card in a Micro Channel machine.  This parameter is a

decimal number in the range of 1 - 10.

MaxRequests     Specifies the number of general request queue entries.

The default is 8.  This parameter is a decimal number

in the range of 6 - 24.

MaxTransmits     Specifies the number of transmit queue entries.  The

default is 12.  This parameter is a decimal number in

the range of 6 - 24.

ReceiveBuffers     Specifies the number of receive buffer queue entries.

The default value is 12.  This parameter is a decimal

number in the range of 6 - 24.

ReceiveBufSize     Specifies the size in bytes of a receive buffer.  The

default value is 256.  This parameter is a decimal

number in the range of 256 - 1536.

ReceiveChains     Specifies the number of receive chain header queue

entries.  The default is 12.  This parameter is a

decimal number in the range of 6 - 24.

6.0	ERROR MESSAGES

No WD8003 Adapters Specified In Protocol.INI Present !

No adapter is installed.  Install a Western Digital Ethernet card.

No PROTOCOL.INI entries found for this driver.  Ensure that a

PROTOCOL.INI entry exists with DriverName = MACWD$ (or MACWD2$).

Checksum of IOBase is incorrect.  Cycle power on the system.

Try configuring the adapter to an alternate IOBase.  If the

problem persists,  consult your adapter vendor.

Protocol.INI parameters do not match the adapter configuration.

POS data or AT bus software configured adapter data does not match

the PROTOCOL.INI entry.  Edit the PROTOCOL.INI file, adjusting the

parameter values to correspond to the adapter configuration.

Conflicting WD8003 Adapter Parameters Specified In Protocol.INI !

More than one PROTOCOL.INI entry is found for this adapter.  Ensure that

DriverName = MACWD$ (or MACWD2$) appears only once in PROTOCOL.INI.

Unknown WD8003 Family Board ID !

The adapter installed is not supported by this driver.

Consult your adapter vendor for more information.

A parameter value in the Protocol.INI file is out of range.

One of the parameters in the PROTOCOL.INI file entry for this adapter

is out of range.  Consult the INSTALL.DOC file you received with the

driver for valid parameter values.

Trademarks:

Microsoft OS/2 LAN Manager is a trademark of Microsoft

Corporation.  3Com, 3+, 3+Open and EtherLink II are registered trademarks

of 3Com Corporation.  EtherCard PLUS, EtherCard PLUS16 StarCard PLUS,

StarLink PLUS and SuperDisk are trademarks of Western Digital Corporation.